Diseño e Implementación de Chips y Circuitos Lógicos

Isabella Cadavid Posada

Clase: Organización de Computadores

Jose Luis Montoya Pareja

Escuela de Ingenierías y Ciencias Aplicadas, Universidad EAFIT

2026

# INTRODUCCIÓN

En el presente documento se describe el diseño e implementación de diferentes chips y circuitos lógicos desarrollados en el entorno Nand2Tetris, así como su respectiva construcción y simulación en Logisim. Se presentan los chips junto con sus circuitos correspondientes, abarcando compuertas básicas, multiplexores, demultiplexores y estructuras aritméticas de 16 bits, fundamentales en la arquitectura de computadores.

El desarrollo de estos componentes permite comprender cómo, a partir de compuertas elementales como NOT, AND y OR, es posible construir estructuras más complejas como multiplexores, sumadores completos y circuitos aritméticos de mayor capacidad. Cada chip fue implementado utilizando únicamente componentes previamente construidos, siguiendo el principio de diseño jerárquico y modular.

- ALU

CHIP ALU {

IN

x[16], y[16],

zx, nx, zy, ny, f, no;

OUT

out[16],

zr,

ng;

PARTS:

// X

Mux16(a=x, b=false, sel=zx, out=xz);

Not16(in=xz, out=nxz);

Mux16(a=xz, b=nxz, sel=nx, out=xf);

// Y

Mux16(a=y, b=false, sel=zy, out=yz);

Not16(in=yz, out=nyz);

Mux16(a=yz, b=nyz, sel=ny, out=yf);

// AND / ADD

And16(a=xf, b=yf, out=andOut);

Add16(a=xf, b=yf, out=addOut);

Mux16(a=andOut, b=addOut, sel=f, out=opOut);

// Negación final

Not16(in=opOut, out=notOut);

Mux16(

a=opOut,

b=notOut,

sel=no,

out[0]=fo0,   out[1]=fo1,

out[2]=fo2,   out[3]=fo3,

out[4]=fo4,   out[5]=fo5,

out[6]=fo6,   out[7]=fo7,

out[8]=fo8,   out[9]=fo9,

out[10]=fo10, out[11]=fo11,

out[12]=fo12, out[13]=fo13,

out[14]=fo14, out[15]=fo15

);

// Conectar salida

Mux16(

a[0]=fo0,   a[1]=fo1,

a[2]=fo2,   a[3]=fo3,

a[4]=fo4,   a[5]=fo5,

a[6]=fo6,   a[7]=fo7,

a[8]=fo8,   a[9]=fo9,

a[10]=fo10, a[11]=fo11,

a[12]=fo12, a[13]=fo13,

a[14]=fo14, a[15]=fo15,

b=false,

sel=false,

out=out

);

// ng

Mux(a=false, b=true, sel=fo15, out=ng);

// zr

Or(a=fo0,  b=fo1,  out=o1);

Or(a=o1,   b=fo2,  out=o2);

Or(a=o2,   b=fo3,  out=o3);

Or(a=o3,   b=fo4,  out=o4);

Or(a=o4,   b=fo5,  out=o5);

Or(a=o5,   b=fo6,  out=o6);

Or(a=o6,   b=fo7,  out=o7);

Or(a=o7,   b=fo8,  out=o8);

Or(a=o8,   b=fo9,  out=o9);

Or(a=o9,   b=fo10, out=o10);

Or(a=o10,  b=fo11, out=o11);

Or(a=o11,  b=fo12, out=o12);

Or(a=o12,  b=fo13, out=o13);

Or(a=o13,  b=fo14, out=o14);

Or(a=o14,  b=fo15, out=oAll);

Not(in=oAll, out=zr);

}

- RAM4K

CHIP RAM4K {

IN in[16], load, address[12];

OUT out[16];

PARTS:

DMux8Way(in=load, sel=address[9..11],

a=loadA, b=loadB, c=loadC, d=loadD,

e=loadE, f=loadF, g=loadG, h=loadH);

RAM512(in=in, load=loadA, address=address[0..8], out=outA);

RAM512(in=in, load=loadB, address=address[0..8], out=outB);

RAM512(in=in, load=loadC, address=address[0..8], out=outC);

RAM512(in=in, load=loadD, address=address[0..8], out=outD);

RAM512(in=in, load=loadE, address=address[0..8], out=outE);

RAM512(in=in, load=loadF, address=address[0..8], out=outF);

RAM512(in=in, load=loadG, address=address[0..8], out=outG);

RAM512(in=in, load=loadH, address=address[0..8], out=outH);

Mux8Way16(a=outA, b=outB, c=outC, d=outD,

e=outE, f=outF, g=outG, h=outH,

sel=address[9..11], out=out);

}

- RAM16K

CHIP RAM16K {

IN in[16], load, address[14];

OUT out[16];

PARTS:

DMux4Way(in=load, sel=address[12..13], a=l1, b=l2, c=l3, d=l4);

RAM4K(in=in, load=l1, address=address[0..11], out=out1);

RAM4K(in=in, load=l2, address=address[0..11], out=out2);

RAM4K(in=in, load=l3, address=address[0..11], out=out3);

RAM4K(in=in, load=l4, address=address[0..11], out=out4);

Mux4Way16(a=out1, b=out2, c=out3, d=out4, sel=address[12..13], out=out);

}

- PC

CHIP PC {

IN in[16], reset, load, inc;

OUT out[16];

PARTS:

Inc16(in=feedback, out=pcPlusOne);

Mux16(a=feedback, b=pcPlusOne, sel=inc, out=outInc);

Mux16(a=outInc, b=in, sel=load, out=outLoad);

Mux16(a=outLoad, b=false, sel=reset, out=outReset);

Register(in=outReset, load=true, out=out, out=feedback);

}