Diseño e Implementación de Chips y Circuitos Lógicos

Isabella Cadavid Posada

Clase: Organización de Computadores

Jose Luis Montoya Pareja

Escuela de Ingenierías y Ciencias Aplicadas, Universidad EAFIT

2026

TABLA DE CONTENIDO

INTRODUCCIÓN	3

1.	NOT	4

2.	AND	5

3.	OR	6

4.	XOR	8

5.	MUX	9

6.	DMUX	11

7.	Not16	12

8.	And16	13

9.	Or16	14

10.	Mux16	15

11.	Or8Way	16

12.	Mux4Way16	17

13.	Mux8Way16	18

14.	DMux4Way	19

15.	DMux8Way	20

16.	Full Adder:	21

17.	HalfAdder	23

18.	FullAdder Resumido:	24

19.	Add16	25

20.	Inc16	26

# INTRODUCCIÓN

En el presente documento se describe el diseño e implementación de diferentes chips y circuitos lógicos desarrollados en el entorno Nand2Tetris, así como su respectiva construcción y simulación en Logisim. Se presentan los chips junto con sus circuitos correspondientes, abarcando compuertas básicas, multiplexores, demultiplexores y estructuras aritméticas de 16 bits, fundamentales en la arquitectura de computadores.

El desarrollo de estos componentes permite comprender cómo, a partir de compuertas elementales como NOT, AND y OR, es posible construir estructuras más complejas como multiplexores, sumadores completos y circuitos aritméticos de mayor capacidad. Cada chip fue implementado utilizando únicamente componentes previamente construidos, siguiendo el principio de diseño jerárquico y modular.

- NOT

CHIP Not {

IN in;

OUT out;

PARTS:

Nand(a= in, b=in , out=out );

}

- AND

CHIP And {

IN a, b;

OUT out;

PARTS:

Nand(a=a , b=b , out=And1 );

Not(in=And1, out=out );

}

- OR

1 Forma

CHIP Or {

IN a, b;

OUT out;

PARTS:

Not(in=a , out=notA );

Not(in=b , out=notB );

Nand(a= notA, b=notB , out= out);

}

2 Forma

CHIP Or {

IN a, b;

OUT out;

PARTS:

Not(in=a , out=notA );

Not(in=b , out=notB );

And(a= notA, b=notB , out= And1);

Not(in=And1 , out=out );

}

- XOR

CHIP Xor {

IN a, b;

OUT out;

PARTS:

Not(in=a , out=notA );

Not(in=b , out=notB );

And(a=notA , b=b , out=And1 );

And(a=a , b=notB , out=And2 );

Or(a=And1 , b=And2 , out=out );

}

- MUX

1 Forma

CHIP Mux {

IN a, b, sel;

OUT out;

PARTS:

Not(in=sel, out=notSel);

Not(in=a,   out=notA);

Not(in=b,   out=notB);

And(a=notSel, b=a, out=and1A);

And(a=and1A,  b=notB, out=and1);

And(a=notSel, b=a, out=and2A);

And(a=and2A,  b=b, out=and2);

And(a=sel,    b=notA,  out=and3A);

And(a=and3A,  b=b,  out=and3);

And(a=sel,    b=a,  out=and4A);

And(a=and4A,  b=b,   out=and4);

Or(a=and1, b=and2, out=or1);

Or(a=and3, b=and4, out=or2);

Or(a=or1,  b=or2,  out=out);

}

2 Forma

CHIP Mux {

IN a, b, sel;

OUT out;

PARTS:

Not(in=sel , out=notSel );

And(a=notSel , b=a , out=and1 );

And(a=sel , b=b , out=and2 );

Or(a=and1 , b=and2 , out=out );

}

- DMUX

CHIP DMux {

IN in, sel;

OUT a, b;

PARTS:

Not(in=sel, out=notS);

And(a=in, b=notS, out=a);

And(a=in, b=sel, out=b);

}

- Not16

CHIP Not16 {

IN in[16];

OUT out[16];

PARTS:

Not(in=in[0] , out=out[0]);

Not(in=in[1] , out=out[1]);

Not(in=in[2] , out=out[2]);

Not(in=in[3] , out=out[3]);

Not(in=in[4] , out=out[4]);

Not(in=in[5] , out=out[5]);

Not(in=in[6] , out=out[6]);

Not(in=in[7] , out=out[7]);

Not(in=in[8] , out=out[8]);

Not(in=in[9] , out=out[9]);

Not(in=in[10] , out=out[10]);

Not(in=in[11] , out=out[11]);

Not(in=in[12] , out=out[12]);

Not(in=in[13] , out=out[13]);

Not(in=in[14] , out=out[14]);

Not(in=in[15] , out=out[15]);

}

- And16

CHIP And16 {

IN a[16], b[16];

OUT out[16];

PARTS:

And(a=a[0],  b=b[0],  out=out[0]);

And(a=a[1],  b=b[1],  out=out[1]);

And(a=a[2],  b=b[2],  out=out[2]);

And(a=a[3],  b=b[3],  out=out[3]);

And(a=a[4],  b=b[4],  out=out[4]);

And(a=a[5],  b=b[5],  out=out[5]);

And(a=a[6],  b=b[6],  out=out[6]);

And(a=a[7],  b=b[7],  out=out[7]);

And(a=a[8],  b=b[8],  out=out[8]);

And(a=a[9],  b=b[9],  out=out[9]);

And(a=a[10], b=b[10], out=out[10]);

And(a=a[11], b=b[11], out=out[11]);

And(a=a[12], b=b[12], out=out[12]);

And(a=a[13], b=b[13], out=out[13]);

And(a=a[14], b=b[14], out=out[14]);

And(a=a[15], b=b[15], out=out[15]);

}

- Or16

CHIP Or16 {

IN a[16], b[16];

OUT out[16];

PARTS:

Or(a=a[0],  b=b[0],  out=out[0]);

Or(a=a[1],  b=b[1],  out=out[1]);

Or(a=a[2],  b=b[2],  out=out[2]);

Or(a=a[3],  b=b[3],  out=out[3]);

Or(a=a[4],  b=b[4],  out=out[4]);

Or(a=a[5],  b=b[5],  out=out[5]);

Or(a=a[6],  b=b[6],  out=out[6]);

Or(a=a[7],  b=b[7],  out=out[7]);

Or(a=a[8],  b=b[8],  out=out[8]);

Or(a=a[9],  b=b[9],  out=out[9]);

Or(a=a[10], b=b[10], out=out[10]);

Or(a=a[11], b=b[11], out=out[11]);

Or(a=a[12], b=b[12], out=out[12]);

Or(a=a[13], b=b[13], out=out[13]);

Or(a=a[14], b=b[14], out=out[14]);

Or(a=a[15], b=b[15], out=out[15]);

}

- Mux16

CHIP Mux16 {

IN a[16], b[16], sel;

OUT out[16];

PARTS:

Mux(a=a[0],  b=b[0],  sel=sel, out=out[0]);

Mux(a=a[1],  b=b[1],  sel=sel, out=out[1]);

Mux(a=a[2],  b=b[2],  sel=sel, out=out[2]);

Mux(a=a[3],  b=b[3],  sel=sel, out=out[3]);

Mux(a=a[4],  b=b[4],  sel=sel, out=out[4]);

Mux(a=a[5],  b=b[5],  sel=sel, out=out[5]);

Mux(a=a[6],  b=b[6],  sel=sel, out=out[6]);

Mux(a=a[7],  b=b[7],  sel=sel, out=out[7]);

Mux(a=a[8],  b=b[8],  sel=sel, out=out[8]);

Mux(a=a[9],  b=b[9],  sel=sel, out=out[9]);

Mux(a=a[10], b=b[10], sel=sel, out=out[10]);

Mux(a=a[11], b=b[11], sel=sel, out=out[11]);

Mux(a=a[12], b=b[12], sel=sel, out=out[12]);

Mux(a=a[13], b=b[13], sel=sel, out=out[13]);

Mux(a=a[14], b=b[14], sel=sel, out=out[14]);

Mux(a=a[15], b=b[15], sel=sel, out=out[15]);

}

- Or8Way

CHIP Or8Way {

IN in[8];

OUT out;

PARTS:

Or(a=in[0], b=in[1], out=or1);

Or(a=in[2], b=in[3], out=or2);

Or(a=in[4], b=in[5], out=or3);

Or(a=in[6], b=in[7], out=or4);

Or(a=or1, b=or2, out=or5);

Or(a=or3, b=or4, out=or6);

Or(a=or5, b=or6, out=out);

}

- Mux4Way16

CHIP Mux4Way16 {

IN a[16], b[16], c[16], d[16], sel[2];

OUT out[16];

PARTS:

Mux16(a=a, b=b, sel=sel[0], out=eleccionAB);

Mux16(a=c, b=d, sel=sel[0], out=eleccionCD);

Mux16(a=eleccionAB, b=eleccionCD, sel=sel[1], out=out);

}

- Mux8Way16

CHIP Mux8Way16 {

IN a[16], b[16], c[16], d[16],

e[16], f[16], g[16], h[16],

sel[3];

OUT out[16];

PARTS:

Mux4Way16(a=a, b=b, c=c, d=d, sel=sel[0..1], out=out1);

Mux4Way16(a=e, b=f, c=g, d=h, sel=sel[0..1], out=out2);

Mux16(a=out1, b=out2, sel=sel[2], out=out);

}

- DMux4Way

CHIP DMux4Way {

IN in, sel[2];

OUT a, b, c, d;

PARTS:

DMux(in=in, sel=sel[1], a=ab, b=cd);

DMux(in=ab, sel=sel[0], a=a, b=b);

DMux(in=cd, sel=sel[0], a=c, b=d);

}

- DMux8Way

CHIP DMux8Way {

IN in, sel[3];

OUT a, b, c, d, e, f, g, h;

PARTS:

DMux(in=in , sel=sel[2] , a=abcd , b=efgh );

DMux(in=abcd, sel=sel[1], a=ab, b=cd);

DMux(in=efgh, sel=sel[1], a=ef, b=gh);

DMux(in=ab, sel=sel[0], a=a, b=b);

DMux(in=cd, sel=sel[0], a=c, b=d);

DMux(in=ef, sel=sel[0], a=e, b=f);

DMux(in=gh, sel=sel[0], a=g, b=h);

}

- Full Adder:

CHIP FullAdder {

IN a, b, c;  // 1-bit inputs

OUT sum,     // Right bit of a + b + c

carry;   // Left bit of a + b + c

PARTS:

Not(in=a , out=notA );

Not(in=b , out=notB );

Not(in=c , out=notC );

And(a=notA , b=notB , out=and1);

And(a=and1 , b=c , out=out1);

And(a=notA , b=b , out=and2);

And(a=and2 , b=notC , out=out2);

And(a=a , b=notB , out=and3);

And(a=and3 , b=notC , out=out3);

And(a=a , b=b , out=and4);

And(a=and4 , b=c , out=out4);

Or(a=out1, b=out2, out=salida1 );

Or(a=out3 , b=out4 , out=salida2 );

Or(a=salida1 , b=salida2 , out=sum );

//Carry

And(a=notA , b=b , out=andCarry1 );

And(a=andCarry1 , b=c , out=outcarry1 );

And(a=a , b=notB , out=andCarry2 );

And(a=andCarry2 , b=c , out=outcarry2 );

And(a=a , b=b , out=andCarry3 );

And(a=andCarry3 , b=notC , out=outcarry3 );

And(a=a , b=b , out=andCarry4 );

And(a=andCarry4 , b=c , out=outcarry4 );

Or(a=outcarry1 , b=outcarry2 , out=salidacarry1);

Or(a=outcarry3 , b=outcarry4 , out=salidacarry2);

Or(a=salidacarry1 , b=salidacarry2 , out=carry);

}

- HalfAdder

CHIP HalfAdder {

IN a, b;

OUT sum, carry;

PARTS:

Xor(a=a, b=b, out=sum);

And(a=a, b=b, out=carry);

}

- FullAdder Resumido:

CHIP FullAdder {

IN a, b, c;

OUT sum,

carry;

PARTS:

Xor(a=a, b=b, out=x);

Xor(a=x, b=c, out=sum);

And(a=a, b=b, out=ab);

And(a=x, b=c, out=xc);

Or(a=ab, b=xc, out=carry);

}

- Add16

CHIP Add16 {

IN a[16], b[16];

OUT out[16];

PARTS:

HalfAdder(a=a[0], b=b[0], sum=out[0], carry=c1);

// bits 1 a 15

FullAdder(a=a[1],  b=b[1],  c=c1,  sum=out[1],  carry=c2);

FullAdder(a=a[2],  b=b[2],  c=c2,  sum=out[2],  carry=c3);

FullAdder(a=a[3],  b=b[3],  c=c3,  sum=out[3],  carry=c4);

FullAdder(a=a[4],  b=b[4],  c=c4,  sum=out[4],  carry=c5);

FullAdder(a=a[5],  b=b[5],  c=c5,  sum=out[5],  carry=c6);

FullAdder(a=a[6],  b=b[6],  c=c6,  sum=out[6],  carry=c7);

FullAdder(a=a[7],  b=b[7],  c=c7,  sum=out[7],  carry=c8);

FullAdder(a=a[8],  b=b[8],  c=c8,  sum=out[8],  carry=c9);

FullAdder(a=a[9],  b=b[9],  c=c9,  sum=out[9],  carry=c10);

FullAdder(a=a[10], b=b[10], c=c10, sum=out[10], carry=c11);

FullAdder(a=a[11], b=b[11], c=c11, sum=out[11], carry=c12);

FullAdder(a=a[12], b=b[12], c=c12, sum=out[12], carry=c13);

FullAdder(a=a[13], b=b[13], c=c13, sum=out[13], carry=c14);

FullAdder(a=a[14], b=b[14], c=c14, sum=out[14], carry=c15);

FullAdder(a=a[15], b=b[15], c=c15, sum=out[15], carry=ignore);

}

- Inc16

1 Forma

CHIP Inc16 {

IN in[16];

OUT out[16];

PARTS:

// El primer bit suma el bit de entrada con 'true' (1)

HalfAdder(a=in[0], b=true, sum=out[0], carry=c0);

// Los siguientes bits suman su entrada con el acarreo anterior

HalfAdder(a=in[1], b=c0, sum=out[1], carry=c1);

HalfAdder(a=in[2], b=c1, sum=out[2], carry=c2);

HalfAdder(a=in[3], b=c2, sum=out[3], carry=c3);

HalfAdder(a=in[4], b=c3, sum=out[4], carry=c4);

HalfAdder(a=in[5], b=c4, sum=out[5], carry=c5);

HalfAdder(a=in[6], b=c5, sum=out[6], carry=c6);

HalfAdder(a=in[7], b=c6, sum=out[7], carry=c7);

HalfAdder(a=in[8], b=c7, sum=out[8], carry=c8);

HalfAdder(a=in[9], b=c8, sum=out[9], carry=c9);

HalfAdder(a=in[10], b=c9, sum=out[10], carry=c10);

HalfAdder(a=in[11], b=c10, sum=out[11], carry=c11);

HalfAdder(a=in[12], b=c11, sum=out[12], carry=c12);

HalfAdder(a=in[13], b=c12, sum=out[13], carry=c13);

HalfAdder(a=in[14], b=c13, sum=out[14], carry=c14);

HalfAdder(a=in[15], b=c14, sum=out[15], carry=ignore);

// El último acarreo (c15) se ignora en un incrementador estándar de 16 bits

}

2 Forma

CHIP Inc16 {

IN in[16];

OUT out[16];

PARTS:

// El incremento es sumar 1. Usamos un sumador de 16 bits.

Add16(a=in, b[0]=true, b[1..15]=false, out=out);

}

- Bit

CHIP Bit {

IN in, load;

OUT out;

PARTS:

Mux(a=outMux , b=in , sel= load , out=DFF1 );

DFF(in=DFF1 , out=out, out=outMux );

}

- Registre

CHIP Register {

IN in[16], load;

OUT out[16];

PARTS:

Bit(in=in[0] , load=load , out=out[0] );

Bit(in=in[1] , load=load , out=out[1] );

Bit(in=in[2] , load=load , out=out[2] );

Bit(in=in[3] , load=load , out=out[3] );

Bit(in=in[4] , load=load , out=out[4] );

Bit(in=in[5] , load=load , out=out[5] );

Bit(in=in[6] , load=load , out=out[6] );

Bit(in=in[7] , load=load , out=out[7] );

Bit(in=in[8] , load=load , out=out[8] );

Bit(in=in[9] , load=load , out=out[9] );

Bit(in=in[10] , load=load , out=out[10] );

Bit(in=in[11] , load=load , out=out[11] );

Bit(in=in[12] , load=load , out=out[12] );

Bit(in=in[13] , load=load , out=out[13] );

Bit(in=in[14] , load=load , out=out[14] );

Bit(in=in[15] , load=load , out=out[15] );

}

- RAM8

CHIP RAM8 {

IN in[16], load, address[3];

OUT out[16];

PARTS:

DMux8Way(in=load, sel=address, a=load0, b=load1, c=load2, d=load3, e=load4, f=load5, g=load6, h=load7);

Register(in=in, load=load0, out=reg0);

Register(in=in, load=load1, out=reg1);

Register(in=in, load=load2, out=reg2);

Register(in=in, load=load3, out=reg3);

Register(in=in, load=load4, out=reg4);

Register(in=in, load=load5, out=reg5);

Register(in=in, load=load6, out=reg6);

Register(in=in, load=load7, out=reg7);

Mux8Way16(a=reg0, b=reg1, c=reg2, d=reg3, e=reg4, f=reg5, g=reg6, h=reg7, sel=address, out=out);

}

- RAM64

CHIP RAM64 {

IN in[16], load, address[6];

OUT out[16];

PARTS:

DMux8Way(in=load, sel=address[3..5], a=l0, b=l1, c=l2, d=l3, e=l4, f=l5, g=l6, h=l7);

RAM8(in=in, load=l0, address=address[0..2], out=out0);

RAM8(in=in, load=l1, address=address[0..2], out=out1);

RAM8(in=in, load=l2, address=address[0..2], out=out2);

RAM8(in=in, load=l3, address=address[0..2], out=out3);

RAM8(in=in, load=l4, address=address[0..2], out=out4);

RAM8(in=in, load=l5, address=address[0..2], out=out5);

RAM8(in=in, load=l6, address=address[0..2], out=out6);

RAM8(in=in, load=l7, address=address[0..2], out=out7);

Mux8Way16(a=out0, b=out1, c=out2, d=out3, e=out4, f=out5, g=out6, h=out7, sel=address[3..5], out=out);

}

25.RAM512

CHIP RAM512 {

IN in[16], load, address[9];

OUT out[16];

PARTS:

DMux8Way(in=load, sel=address[6..8], a=l1, b=l2, c=l3, d=l4, e=l5, f=l6, g=l7, h=l8);

RAM64(in=in, load=l1, address=address[0..5], out=r1);

RAM64(in=in, load=l2, address=address[0..5], out=r2);

RAM64(in=in, load=l3, address=address[0..5], out=r3);

RAM64(in=in, load=l4, address=address[0..5], out=r4);

RAM64(in=in, load=l5, address=address[0..5], out=r5);

RAM64(in=in, load=l6, address=address[0..5], out=r6);

RAM64(in=in, load=l7, address=address[0..5], out=r7);

RAM64(in=in, load=l8, address=address[0..5], out=r8);

Mux8Way16(a=r1, b=r2, c=r3, d=r4, e=r5, f=r6, g=r7, h=r8, sel=address[6..8], out=out);

}